SECTION 06107 - EXTERIOR SIDING & ENCLOSURE - LEAD-BASED PAINT

PART 1 - GENERAL

1.1		RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract, including

General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

1.2		SUMMARY OF WORK:

A.	Work under this section includes the furnishing of all

labor, materials, and equipment required to enclose leadbased painted exterior surfaces with the following:

1.  Aluminum clapboard siding.

2.  Vinyl clapboard siding.

3.  Lumber siding

PART 2 - PRODUCTS

2.1		ENCLOSURE SYSTEM MATERIALS:

A.	Aluminum Clapboard Siding:  Formed aluminum siding and

accessories complying with AAMA 1402 fabricated from aluminum sheet.

B.	Vinyl Clapboard Siding: Solid vinyl siding and accessories

fabricated by extrusion from a polyvinyl chloride compound complying with ASTM B-3679.

C.	Lumber Siding:    Kiln dried wood lumber for siding in size

and pattern shown on drawings.

D.	Siding Accessories: Solid soffit panels, ventilating soffit

panels, post, door castings, window casings, starter strips, and field fabricated materials to match type of siding selected.

E.	Fasteners: Non-corrosive aluminum siding nails of sufficient

length to penetrate substrate 1" (25 mm).  Stainless steel, non-corrosive aluminum or hot dipped galvanized are to be used for wood lumber siding.

F.	Air Infiltration Barrier: House wrap and compatible seaming

tape.

G.	Insulation:  Rigid or fan-fold insulation intended as a

substrate for siding materials.

PART 3 - EXECUTION

3.1		PREREQUISITES

A.	Before starting work of this section, complete the

following:

1.  Section 01505 - Exterior Regulated Areas

2   Section 01555 - Worker Protection

1.  Section 01556 - Respiratory protection

3.2		PREPARATION

A.	Prior to installation of enclosure carefully remove flaking

and loose areas of lead-based paint by wet scraping.  Allow paint with good adhesion to remain.

B.	Affix 3' x 5" (76x127 mm) warning labels every 4 feet (1.22

m) vertically and 4 feet (1.22 m) horizontally, for a minimum of one label every 16 square feet, on surface reading 'Lead-Based Paint" prior to installing enclosure or using an indelible marker write "Lead-Based Paint" every 4 feet (1.22 m).

3.3		INSTALLATION

A.	Comply with siding manufacturer's installation instructions

and recommendations.  Install rigid sheathing or furring strips to provide a level surface prior to installation of siding.  Do not remove original siding.

B.	Wrap existing siding with air infiltration barrier or

install rigid or fan fold insulation prior to installation of siding.  Install infiltration barrier and/or insulation so that it forms a continuous layer under the siding and becomes a barrier to the release of dust through cracks in the siding.

C.	Install siding and accessories to cover all lead-based

painted materials.  Field fabricate accessories that are not commercially available from sheet aluminum stock or wood. Caulk all joints and seams.

END OF SECTION 06107