SECTION 06106 - WOOD ENCLOSURES - LEAD-BASED PAINT

PART 1 - GENERAL

1.1		RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract, including

General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

1.2		SUMMARY

A.	This Section includes the furnishing of all labor,

materials, and equipment required for wood enclosures of lead-based painted surfaces.

B.	Related Sections:  The following Sections contain

requirements that relate to this Section:

1.  Section 01505 - Exterior Regulated Areas

2.  Section 01555 - Worker Protection

3.  Section 01556 - Respiratory protection

4.  Division 6 Section "Rough Carpentry" for furring, blocking, and other carpentry work that is not exposed to view.

5.  Division 6 Section "Finish Carpentry" for carpentry exposed to view that is not specified in this section.

6.  Division 6 Section "Exterior Architectural Woodwork" for exterior woodwork.

7.  Division 8 Section "Flush Wood Doors" for doors specified by reference to architectural woodwork standards.

8.  Division 8 Section "Stile and Rail Wood Doors" for doors specified by reference to architectural woodwork standards.

9.  Division 8 Section "Wood Windows" for stock wood

windows.

10. Division 9 Section "Painting" for final finishing of installed architectural woodwork.

1.3		DELIVERY, STORAGE, AND HANDLING

A.	Protect woodwork during transit, delivery, storage, and

handling to prevent damage, soilage, and deterioration.

B.	Do not deliver wood for enclosures until painting, wet work,

grinding, and similar operations that could damage, soil, or deteriorate woodwork have been completed in installation areas.  If wood must be stored in other than installation areas, store only in areas whose environmental conditions meet requirements specified in "Project Conditions."

1.4		PROJECT CONDITIONS

A.	Environmental Conditions:  Do not install wood enclosures

until the interior is protected from the weather and is heated, if required, to 60 degrees fahrenheit.

B.	Field Measurements:  Field fit wood enclosures to existing

construction.

PART 2 - PRODUCTS

2.1		FABRICATION, GENERAL

A.	Wood Moisture Content:  Comply with requirements of

referenced quality standard for moisture content of lumber in relation to relative humidity conditions existing during time of fabrication and in installation areas.

B.	Fabricate woodwork to dimensions, profiles, and details

indicated. Ease edges to radius indicated for the following: 1.  Corners of cabinets and edges of solid wood (lumber)

members less than 1 inch in nominal thickness: 1/16 inch.

2.  Edges of rails and similar members more than 1 inch in nominal thickness: 1/8 inch.

2.2		 JOB FABRICATED WOOD ENCLOSURES AND TRIM FOR OPAQUE FINISH

A.	Quality Standard:  Comply with AWI Section 300.

B.	Quality Standard:  Comply with WIC Section 10 "Interior

Trim."

C.	Grade:

1.  Premium.

2.  Custom.

3.  Economy.

D.	Backout or groove backs of flat trim members and kerf backs

of other wide flat members, except for members with ends exposed in finished work.

E.	Lumber Species:  Any closed-grain hardwood listed in

referenced woodworking standard.

F.	Lumber Species:  Eastern white pine, sugar pine, or Idaho

white pine.

2.3		FASTENERS AND ANCHORS

A.	Screws:  Select material, type, size, and finish required

for each use.  Comply with FS FF-S-111 for applicable requirements.

1.  For metal framing supports, provide screws as recommended by metal framing manufacturer.

B.	Nails:  Select material, type, size, and finish required for

each use.  Comply with FS FF-N-105 for applicable requirements.

C.	Anchors:  Select material, type, size, and finish required

by each substrate for secure anchorage.  Provide nonferrous metal or hot-dip galvanized anchors and inserts on inside face of exterior walls and elsewhere as required for corrosion resistance.  Provide toothed steel or lead expansion bolt devices for drilled-in-place anchors. Furnish inserts and anchors, as required, to be set into concrete or masonry work for subsequent woodwork anchorage.

PART 3 - EXECUTION

3.1		GENERAL

A.	Before starting work of this section, complete the

following:

1.  Section 01505 - Exterior Regulated Areas

2.  Section 01555 - Worker Protection

3.  Section 01556 - Respiratory protection

3.2		PREPARATION

A.	Prior to installation of enclosure carefully remove flaking

and loose areas of lead-based paint by wet scraping.  Allow paint with good adhesion to remain.

B.	Affix 3" x 5" (76 x 127 mm) warning labels every 4 feet

(1.22 m) on surface reading "Lead-Based Paint" or using an indelible marker write "Lead-Based Paint" every 4 feet (1.22 m) prior to installing air infiltration barrier or siding .

C.	Clean substrates of projections and substances detrimental

to application.

D.	Condition wood to average prevailing humidity conditions in

installation areas before installing.

3.3		INSTALLATION

A.	Seal Edges: Calk all joints and seal all edges of wood

enclosures.  Apply caulk to edges of trim that abut existing construction.  Back caulk edges of any panels that are not sealed with trim.

B.	Back-paint:  exterior wood trim prior to installation.

C.	Install wood trim plumb, level, true, and straight with no

distortions.  Shim as required with concealed shims. Install to a tolerance of 1/8 inch in 8'-0" for plumb and level (including tops) and with no variations in flushness of adjoining surfaces.

D.	Scribe and cut woodwork to fit adjoining work and refinish

cut surfaces or repair damaged finish at cuts.

E.	Anchor woodwork to anchors or blocking built in or directly

attached to substrates.  Secure to grounds, stripping and blocking with countersunk, concealed fasteners and blind nailing as required for a complete installation.  Except where pre-finished matching fastener heads are required, use fine finishing nails for exposed nailing, countersunk and filled flush with woodwork and matching final finish where transparent finish is indicated.

F.	Standing and Running Trim and Rails:  Install with minimum

number of joints possible, using full-length pieces (from maximum length of lumber available) to the greatest extent possible.  Stagger joints in adjacent and related members. Cope at returns and miter at corners.

G.	Paneling:  Anchor paneling to supporting substrate with

concealed panel-hanger clips and by blind nailing on backup strips, splined-connection strips, and similar associated trim and framing. Do not face nail unless otherwise

indicated.

3.4		ADJUSTMENT AND CLEANING

A.	Repair damaged and defective woodwork where possible to

eliminate defects functionally and visually; where not possible to repair, replace woodwork.  Adjust joinery for uniform appearance.

B.	Clean woodwork on exposed and semi-exposed surfaces.  Touch

up factory-applied finishes to restore damaged or soiled areas.

3.5		PROTECTION

A.	Provide final protection and maintain conditions, in a

manner acceptable to manufacturer and installer, that ensures that woodwork is without damage or deterioration at time of Substantial Completion.

END OF SECTION 06106