SECTION 02066 - REMEDIATION OF LEAD CONTAMINATED SOIL

PART 1 - GENERAL

1.1	Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 1 Specification Sections, apply to work of this section.

1.2	Summary Of Work:

A.	Bare soil with child contact, lead concentration  400 PPM up to 1,999 PPM.

1.	Install Sod over bare soil

- Install ground cover and/or shrubbery

B.	Bare soil, with or without child contact, lead concentration - 2,000 PPM up to 4,999PM.

1.	Install sod over bare soil

2.	Install ground cover and/or shrubbery.

3.	Remove soil to a depth of three (3) inches.

C.	Bare soil, lead concentration greater than 5,000 PPM

1.	Remove soil to a depth of three (3) inches.

2.	Install asphalt paving

3.	Install concrete paving

PART 2 - PRODUCTS  (NOT APPLICABLE)

PART 3 - EXECUTION

3.1	SOIL REMOVAL

A.	Use hand held spray equipment to dampen soil.  Do not over saturate and cause water to run-off to adjacent areas.

B.	Remove soil using shovels or vacuum loading equipment starting at the point farthest from load out area.

C.	Load soil into containers, trucks or dumpsters.  Do not track through areas where soil has been removed.  Cover containers, trucks or dumpsters prior to transport.  Dispose of soil  in accordance with section 02067 - Disposal of Waste Materials - Lead-Based Paint .

D.	At the end of each shift or during winds which may disturb and spread soil contamination, cover contaminated sections of soil with 6 mil polyethylene sheeting.

E.	Wash tools, and equipment which came in contact with contaminated soil.  Collect  and filter wash water.  Dispose of in accordance with section 02067 - Disposal of Waste Materials - Lead-Based Paint .

3.2	CLEARANCE CRITERIA

A.	Complete Soil Sampling in accordance with Section 01421 - Work Area Clearance following soil removal.

1.	If release criteria is not met, continue Soil Remediation from that point.

END OF SECTION - 02066