SECTION 02067 - DISPOSAL OF WASTE MATERIALS - LEAD-BASED PAINT

PART 1 - GENERAL

1.1	RELATED DOCUMENTS:

A.	Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division-1 Specification Sections, apply to work of this section.

B.	Section 01094 Codes and Regulations - Lead-Based Paint describes applicable federal, state and local regulations.

1.2	DESCRIPTION OF THE WORK:

A.	This section describes the disposal of lead-containing or lead contaminated waste materials.  Disposal includes packaging of waste materials.  Disposal is accomplished by landfilling.

1.3	SUBMITTALS:

A.	Before Start of Work:  Submit the following to the Designer for review.  Do not start work until these submittals are returned with Designer action stamp indicating that the submittal is returned for unrestricted use.

1.	Contractor must ascertain that the facility owner is registered with the U.S. EPA as a generator of hazardous waste.  If there is no generator status established, the contractor shall assist the owner in obtaining generator identification numbers.

2.	Copy of state or local license for waste hauler.

3.	U.S. EPA identification number of waste hauler.

4.	Name and address of waste disposal facility  where hazardous waste materials are to be disposed.  Include contact person and telephone number.  Copy of state license and permit.  Provide disposal  facility permits.

5.	Copy of EPA "uniform hazardous waste manifest" form.

6.	Copy of EPA "notification of hazardous waste activity" form.

7.	Copy of forms required by state or local agencies.

8.	Sample of disposal bag and labels to be used.

B.	Submit copies of all manifests and disposal site receipts to Owner's project monitor.

PART 2 - PRODUCTS:

2.1	Disposal:  Provide 6 mil thick leak-tight polyethylene bags or wrap components in 6 mil polyethylene sheeting and seal with duct tape.  Label with text as follows:

A.	"Label with specific Hazardous Waste Label: "

B.	For wrapped materials provide stick-on labels.

PART 3 - EXECUTION

3.1	GENERAL:

A.	Contact  DOT, EPA,  state and local authorities to determine lead-based paint disposal requirements.

B.	Testing of waste shall be performed by a laboratory accredited  by either the American Industrial Hygiene Association (AIHA) or the American Association of Laboratory Accrediation (AALA) retained by the contractor.  Include the cost of testing in the contract sum and supply all test results to the owner.

C.	Testing of waste will be performed by a qualified laboratory retained by the owner.  Results will be supplied to the contractor.  The contractor shall pay for any additional samples obtained at the site for his use.

D.	Testing of waste  by use of the Toxicity Characteristic Leaching Procedure (TCLP) test was completed by the Owner prior to this project.  Results will be supplied to the Contractor and can be used in conjunction with samples obtained and analyzed by the Contractor.  The Contractor shall pay for any additional TCLP samples obtained at the site for his use.

E.	Waste tested which results in a lead content in the leachate of greater than or equal to five parts per million is to be considered hazardous, handled and disposed of according to local, city, state, and federal regulations.

F.	Place all waste generated during the project in 6 mil disposal bags or wrap in 6 mil polyethylene sheeting, store in the designated storage area, enclosed dumpsters or trucks.  Separate waste materials into the following categories and label all disposal bags and wrapped packages.

G.	Non-Hazardous Solid Waste:

1.	Baseboard and other wood work.

2.	Plastic sheeting and duct tape used during abatement.

3.	Porch ceiling and trim.

4.	Plaster.

5.	Doors.

H.	Hazardous Solid Waste:

1.	Thresholds.

2.	Paint chips.

3.	Rags, sponges, mops, HEPA Vac filters and contents, respirator cartridges and other materials used during abatement.

I.	Hazardous Liquid Waste (as determined by testing)

1.	Caustic Pastes

2.	Neutralizers

3.	Paints, Solvents

J.	Properly store and secure waste at all times.  Do not leave debris in the yard or in uncovered or unlocked trucks or dumpsters.  Do not incinerate debris or use an unauthorized dumpster.  Do not introduce lead contaminated water into storm or sanitary sewers.  Do not permit recycling of building components coated with Lead-Based Paint.

3.2	DISPOSAL OF NON-HAZARDOUS SOLID WASTE:  (As Determined By Testing)

A.	Materials are to remain in 6 mil disposal bags or wrapped in polyethylene sheeting.  Label all packages.  Substrates removed with paint in good condition which is adhered to the substrate may be placed directly in dumpsters then covered.

B.	Transport waste in covered or enclosed trucks or dumpsters.

3.3	DISPOSAL OF NON-HAZARDOUS LIQUID WASTE:  (As Determined By Testing)

A.	Dispose of liquid waste by pouring into sanitary sewage system if permission is received from publicly owned treatment works facility (POTW).  Do not dispose of liquid waste by pouring onto ground or into storm drain.  If the liquid waste contains phosphates or other chemicals advise treatment facility of quantity of liquid and that it likely will contain phosphates.

3.4	DISPOSAL OF HAZARDOUS LIQUID OR SOLID  WASTES: (As Determined By Testing)

A.	Comply with RCRA, DOT, STATE and local regulations.

B.	Apply for an EPA identification number from the appropriate regional office if more than 100 kg of hazardous waste is generated from the lead hazard reduction process during any calendar month.

C.	Comply with DOT and STATE regulations for containers.  The most stringent regulation shall apply.

D.	All waste is to be hauled by a licensed waste hauler with all required licenses form all state and local authorities with jurisdiction.

E.	Load all waste material into properly labeled disposal bags, polyethylene sheeting, or leak-tight drums.  All materials are to be contained in one of the following:

1.	One 6 mil layer of sheet polyethylene, duct tape all seams or

One 6 mil disposal bag or

2.	Two 4 mil disposal bags

3.	Sealed steel drum with no bag

F.	Protect interior of truck or dumpster with two layers of 6 mil polyethylene sheeting with all seams sealed with duct tape.

G.	Carefully load containerized waste in fully enclosed dumpsters, trucks or other appropriate vehicles for transport.  Exercise care before and during transport, to insure that no unauthorized persons have access to the material.

H.	Do not store containerized materials outside of the Work Area.  Take containers from the Work Area directly to the designated storage area, sealed truck or dumpster,

I.	At disposal site unload containerized waste:

1.	At a disposal site, sealed plastic bags may be carefully unloaded from the truck.  If bags are broken or damaged, return to work site for rebagging.  Clean entire truck and contents using procedures set forth in section 01715 Project Decontamination.

J.	Retain all documents from the disposal site.

K.	At completion of hauling and disposal of each load submit copy of Uniform Hazardous Waste Manifest, to Owner's Project Monitor.

3.5	BACKCHARGES:	Where Contractor fails to fulfill packaging, handling, or disposal requirements as outlined herein, Owner will charge back to Contractor all costs associated with insuring that hazardous wastes are packaged and segregated in accordance with EPA and DOT regulations.

A.	Environmental pollution of Owner's property resulting from Contractor's hazardous waste management activities shall be promptly remediated under Owner direction, to the Owner's sole satisfaction, and at the Contractor's sole expense.

B.	Contractor agrees to either reimburse the Owner, or reduce the Contract amount by change order to cover all costs associated with waste repackaging, waste re-segregation, or pollution remediation efforts.

END OF SECTION - 02067