SECTION  01715 - PROJECT DECONTAMINATION - LEAD-BASED PAINT

PART 1 - GENERAL

1.1	RELATED DOCUMENTS:

A.	Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division-1 Specification sections, apply to work of this section.

1.2	DESCRIPTION OF REQUIREMENTS:.

A.	General:  Decontamination of the Work Area following Lead-Based Paint Hazard Reduction.

1.3	RELATED WORK SPECIFIED ELSEWHERE:

A.	Removal of Gross Debris is integral with the performance of Lead Hazard reduction work and as such is specified in the appropriate work section(s) of these specifications:

1.	Section 02065 Removal of Lead-Based Painted Substrates

2.	Section 05582 Sheet Metal Enclosure - Lead-Based Paint

3.	Section 06106 Wood Enclosures - Lead-Based Paint

4.	Section 06107 Exterior Siding & Enclosure - Lead-Based Paint

5.	Section 06402 Carpentry & Trim Work - Lead-Based Paint

6.	Section 06403 Repair of Lead-Based Painted Substrates

7.	Section 09252 Gypsum Wallboard Enclosure - Lead-Based Paint

8.	Section 09253 Cementitious Enclosures - Lead-Based Paint

9.	Section 09940 Encapsulation - Lead-Based Paint

10.	Section 09951 Chemical Stripping of Lead-Based Paint

11.	Section 09952 Mechanical Removal of Lead-Based Paint

12.	Section 09953 Surface Preparation-Paint Stabilization - Lead-Based Paint

13.	Section 09954 Painting - Lead-Based Paint

B.	Work Area Clearance:  Wipe sample testing and other requirements which must be met before release of Contractor and reoccupancy of the work area are specified in Section 01421 Project Clearance.

PART 2 - PRODUCTS

2.1	Disposal Bags/Plastic Sheeting:  Provide 6 mil polyethylene disposal bags sealed with duct 	tape.

2.2	Wet Detergent Wash:  Provide detergent with high phosphate content (at least 5%) trisodium phosphate (TSP).  Follow dilution ratio recommended by manufacturer's instructions.

Wet Detergent Wash:  Provide detergent or cleaning agent formulated to be effective in removing lead dust.  Follow dilution ratio recommended by the manufacturer's instructions.

PART 3 - EXECUTION

3.1	GENERAL:

A.	Work of This Section:  includes the decontamination of surfaces in the Work Area which has been, or may have been, contaminated by lead dust generated during Hazard Reduction activities, or which may previously have been elevated.

B.	Work of This Section:  includes the cleaning, decontamination, and removal of temporary facilities installed prior to Hazard Reduction work, including:

1.	Floor Sheeting and Critical barriers erected by work of Section 01506

C.	Work of This Section: includes the cleaning, and decontamination of all surfaces (ceiling, walls, floor) of the Work Area, and all cabinetry or equipment in the Work Area.

3.2	START OF WORK:

A.	Previous Work: During completion of the Lead Hazard reduction work specified in other sections, the layer of polyethylene sheeting will have been removed and disposed of along with any gross debris generated by the lead work.

B.	Start of Work: Work of this section begins with the cleaning of the building surfaces.  At start of work the following will be in place:

1.	Floor Sheeting And Critical Barrier:  A barrier between the work area and other portions of the building or the outside.

2.	Critical Barrier Sheeting:  Over lighting fixtures, ventilation, openings, doorways, convectors, and other openings.

3.	Flapped Doorway Sheeting:  Between the work area and  the changing room.

3.3	PRELIMINARY FIRST CLEANING:

A.	Preliminary Cleaning:  Clean tools, scaffolding, ladders and equipment by HEPA vacuuming.  Follow HEPA vacuuming with wet cleaning of all tools and equipment.

B.	Immediately following preliminary cleaning, mist and remove poly sheeting.  Remove highest sheeting first and work down to floor.  Fold sheeting inward to trap any leaded dust or residue.  Place sheeting in 6 mil disposal bags and dispose if in accordance with Section 02067.

3.4	FINAL CLEANING:

A.	HEPA Vacuum:  All surfaces in work area.  Start at point farthest from main entrance and finish vacuuming back at that point.  Begin at top of each room and work down.  Sequence to avoid passing through rooms already cleaned.

B.	Mist Critical barriers sheeting and remove.

C.	HEPA Vacuum area previously covered by critical barrier sheeting.

D.	Perform wet detergent wash of all surfaces.  Begin at point farthest from main entrance, work from top to bottom.  Take care not to damage existing finishes and surfaces.  Change cleaning mixture in accordance with manufacturer's recommendations or minimum after each room.  Filter all waste water or dispose of in accordance with section 02067.

E.	Wiping Work Area

1.	The work area should be cleaned using a three container method.  Fill two buckets with clean water and place them in the work area with the container of cleaning solution.

2.	Pour cleaning solution onto a clean cloth.  Wring excess solution into one of the buckets without placing the cloth into the bucket.  Wipe the work surface with the cloth.  Add more cleaning solution to the cloth and continue wiping until the entire surface area has been covered.  Discard all cloths used in this procedure in the disposal bag.

3.	Dip and wring out a clean cloth in the first rinse bucket.  Wipe off the work area.  Rinse the cloth in the first bucket again and wring out thoroughly.  Rinse the cloth in the second bucket and wring out thoroughly again.

4.	Continue to clean the work surface with the cloth and rinse using this procedure until the entire work surface has been cleaned.  Wipe off all tools to remove any dust.

5.	NOTE:  The rinse water in the bucket should be changed periodically.  The frequency will vary depending on the level  of contamination.

F.	Mopping Work Area

1.	Collect any visible debris using wet cloths before mopping the area.  Pour the cleaning solution into the mop bucket.  fill two rinse buckets with clean water.  Place the mop into the cleaning solution.  Wring excess solution into the mop bucket.  mop small sections of the work area.  Place the mop into the cleaning solution and wring thoroughly between sections.  After the entire surface has been mopped thoroughly, rinse the mop head.  Completely rinse the surface by placing the mop in the first bucket, wringing it out thoroughly, placing it in the second bucket, wringing thoroughly and then mopping the surface.  Continue this cycle until all areas have been rinsed.

2.	NOTE:  The water in the two containers should be changed periodically.  The frequency will depend on the level of contamination.

G.	Perform clear water wash of all surfaces in same manner as wet detergent wash.

H.	After all surfaces in work area are allowed to dry, complete final HEPA vacuuming of all surfaces in same manner as first HEPA vacuuming.

I.	After Final Cleaning Perform a Complete Visual Inspection of the entire work area including:  all surfaces, ceiling, walls, floor, doorways, windows, surfaces previously covered with critical barrier sheeting,  and other openings; look for debris from any source, residue on surfaces, dust or other matter.  If any debris, residue, dust or other matter is found repeat final cleaning and continue decontamination procedure from that point.  When the area is visually clean, complete the certification at the end of this section.  Visual inspection is not complete until confirmed in writing, on the certification, by Project Monitor.

J.	Painting of substrates:  Perform painting/sealing of substrates at this time.

K.	Perform final clearance sampling in accordance with sub-section 3.6 final clearance 		sampling.

3.5	EXTERIOR ABATEMENT DECONTAMINATION:

A.	First Cleaning:  Carry out a first cleaning of all surfaces affected by the work including remaining sheeting, ladders, scaffolding and exterior of building in a ten feet (10') radius from the work by use of a High Efficiency Particulate Air (HEPA) filtered vacuum.  Immediately following first cleaning remove plastic sheeting and dispose of.  Remove any remaining paint chips on soil or surrounding walk ways, porches, etc.

B.	Final Cleaning:  Carry out a final cleaning of all surfaces in the same manner as the first cleaning.  Comply with requirements of visual inspection.

3.6	FINAL CLEARANCE SAMPLING:

A.	Wipe Sampling by Atomic Absorption Spectroscopy (AAS) or Inductively Coupled Plasma Emission Spectroscopy (ICP):   After the work area is found to be visually clean, wipe samples will be obtained and analyzed in accordance with the procedure set forth in Section 01421 Project Clearance.

1.	If Release Criteria are not met, repeat HEPA vacuuming, wet wash, HEPA vacuuming until satisfactory clearance results are obtained.

2.	If Release Criteria are met, remove work area isolation.  Remove all equipment, materials from the site.

B.	Soil Sampling by Atomic Absorption Spectroscopy (AAS) or Inductively Coupled Plasma Emission Spectroscopy (ICP):   After the work area is found to be visually clean, soil samples will be obtained and analyzed in accordance with the procedure set forth in Section 01421 Project Clearance.

3.7	SUBSTANTIAL COMPLETION OF HAZARD REDUCTION:

A.	Hazard Reduction Work is Substantially Complete upon meeting the requirements of this section, section 01421 project Clearance , including submission of:

1.	Certificate of Visual Inspection

2.	Receipts Documenting proper disposal as required by Section 02067 Disposal of Waste Material.

3.	Punch list detailing repairs to be made and incomplete items.

3.8	CERTIFICATE OF VISUAL INSPECTION:

A.	Following this section is a "Certificate of Visual Inspection".  This certification is to be completed by the Contractor and certified by the Project Monitor.  Submit completed certificate with application for final payment.  Final payment will not be made until this certification is executed.

END OF SECTION - 01715

CERTIFICATION OF VISUAL INSPECTION

In accordance with Section 01715 "Project Decontamination" the contractor hereby certifies that he has visually inspected the work area (all surfaces including pipes, counters, ledges, walls, ceiling and floor, behind critical barriers, sheet plastic, etc.) and has found no dust, debris or residue.

by: (Signature______________________________  Date_______________

(Print Name)________________________________

(Print Title)_________________________________

PROJECT MONITOR CERTIFICATION

The Project Monitor hereby certifies that he has accompanied the contractor on his visual inspection and verifies that this inspection has been thorough and to the best of his knowledge and belief, the contractor's certification above is a true and honest one.

by: (Signature______________________________  Date_______________

(Print Name)________________________________

(Print Title)_________________________________

WORK AREA

Location:

Room:

Hazard Reduction Performed: