SECTION 08 41 13 ALUMINUM – FRAMED ENTRANCES and STOREFRONTS

System 406 Thermal Flush-Glazed Screw Spline Storefront

PART 1 GENERAL

1.01	Work Included

A.	Furnish and install aluminum architectural storefront system complete with hardware and related components as shown on drawings and specified in this section.

B.	All storefront systems shall be EFCO® System 406 Thermal Flush-Glazed Screw Spline Storefront. Other manufacturers requesting approval to bid their product as an equal must submit the following information fifteen days prior to close of bidding.

1.	A sample storefront system (size and configuration) as per requirements of architect.

2.	Test reports documenting compliance with requirements of Section 1.05.

C.	Glass

1.	Reference Section 08 81 00 for Glass and Glazing.

D.	Single Source Requirement

1.	All products listed in Section 1.02 shall be by the same manufacturer.

1.02	Related Work

List work and materials related to this section but specified in other sections. EFCO recommends that one manufacturer furnish all applicable Division 8 products to ensure consistent product quality and compatibility.

A.	Section 08 32 13 – Sliding Aluminum – Framed Glass Doors

B.	Section 08 44 13 – Glazed Aluminum Curtain Walls

C.	Section 08 44 33 – Sloped Glazing Assemblies

D.	Section 08 51 13 – Aluminum Windows

E.	Section 10 71 13 – Exterior Sun Control Devices

1.03	Items Furnished but Not Installed

1.04	Items Installed but Not Furnished

1.05	Laboratory Testing and Performance Requirements

A.	Test Units

1.	Air, water, and structural test unit size shall be a minimum of two stories high and three lites wide.

2.	Thermal test unit sizes shall be 80" (2032 mm) wide x 80" (2032 mm) high with one intermediate vertical mullion and two lites of glass.

B.	Test Procedures and Performance

1.	Air Infiltration Test

a.	Test unit in accordance with ASTM E 283 at a static air pressure difference of 6.24 psf
(299 Pa).

b.	Air infiltration shall not exceed .06 cfm/SF (.30 l/s•m²) of unit.

2.	Water Resistance Test

a.	Test unit in accordance with ASTM E 331.

b.	There shall be no uncontrolled water leakage at a static test pressure of 10.0 psf 
(478 Pa).

3.	Uniform Load Deflection Test

a.	Test in accordance with ASTM E 330.

b.	Deflection under design load shall not exceed L/175 of the clear span.

4.	Uniform Load Structural Test

a.	Test in accordance with ASTM E 330 at a pressure 1.5 times the design wind pressure in 1.05.B.3.b.

b.	At conclusion of the test, there shall be no glass breakage, permanent damage to fasteners, storefront parts, or any other damage that would cause the storefront to be defective.

5.	Condensation Resistance Test (CRF)

        - Test unit in accordance with AAMA 1503.1.

        - Condensation Resistance Factor (CRF) shall not be less than ___ (frame) when glazed with ___ center of glass U-Factor. (See chart at end of section).

6.	Condensation Resistance (CR)

a.	With ventilators closed and locked, test unit in accordance with NFRC 500-2010.

b.	Condensation Resistance (CR) shall not be less than ___ when glazed with ___ center of glass U-Factor. (See chart at end of section).

7.	Thermal Transmittance Test (Conductive U-Factor)

            - With ventilators closed and locked, test unit in accordance with NFRC 100-2010.

            - Conductive thermal transmittance (U-Factor) shall not be more than ___ BTU/hr•ft2•°F
( ___ W/m²•K) when glazed with ___ center of glass U-Factor. (See chart at end of section).

| Glass Comparison Chart |  |  |  |  |
| --- | --- | --- | --- | --- |
| Glass | C.O.G.2 U-Factor | U-Factor1 | Frame CRF3 | CR1 |
| 1” IG | 0.47 | 0.61 BTU/hr•ft²•ºF (3.46 W/m²•K) | 68 | * |
| 1” IG | 0.29 | 0.47 BTU/hr•ft²•ºF (2.66 W/m²•K) | 68 | * |
| 1” IG | 0.24 | 0.43 BTU/hr•ft²•ºF (2.44 W/m²•K) | 68 | * |

*Please consult EFCO Product Tech Support department for values.

1U-Factor and Condensation Resistance (CR) are based on a nominal size of 47.25" (1200 mm) x 59" (1500 mm) with two lights of glass using NFRC-100, and 500 - 2010.  2Intercept® Spacer.  3Based on AAMA 1503.1

C.	Project Wind Loads

Architect to specify design wind pressure, both positive and negative required by the governing building code, as calculated from ANSI A58.1, or as determined by boundary layer wind tunnel testing. Use 20.0 psf (958 Pa) as minimum.

1.	The system shall be designed to withstand the following loads normal to the plane of the wall:

a.	Positive pressure of   ___ psf ( ___ Pa) at non-corner zones.

b.	Negative pressure of ___ psf ( ___ Pa) at non-corner zones.

c.	Negative pressure of ___ psf ( ___ Pa) at corner zones.

1.06	Field Testing and Performance Requirements

Insert at discretion of specifier.

A.	Test in accordance with AAMA 501.2 for spray test only or AAMA 503.92 for pressurized test.

1.07	Quality Assurance

A.	Provide test reports from AAMA accredited laboratories certifying the performance as specified in 1.05.

B.	Test reports shall be accompanied by the storefront manufacturer’s letter of certification stating that the tested storefront meets or exceeds the referenced criteria for the appropriate storefront type.

1.08	References

1.09	Submittals

- Contractor shall submit shop drawings; finish samples, test reports, and warranties.

  - Samples of materials as may be requested without cost to owner, i.e., metal, glass, fasteners, anchors, frame sections, mullion section, corner section, etc.

B.	An NFRC Component Modeling Approach (CMA) generated label certificate shall be provided by the manufacturer. The label certificate shall be project specific and will contain the thermal performance ratings of the manufacturer’s framing combined with the specified glass, and the glass spacer used in the fabrication of the glass, at NFRC standard test size as defined in table 4-3 in NFRC 100-2010.

1.10	Warranties

A.	Total Storefront Installation

1.	The responsible contractor shall assume full responsibility and warrant for one year the satisfactory performance of the total storefront installation. This includes the glass (including insulated units), glazing, anchorage and setting system, sealing, flashing, etc., as it relates to air, water and structural adequacy as called for in the specifications and approved shop drawings.

2.	Any deficiencies due to such elements not meeting the specifications shall be corrected by the responsible contractor at their expense during the warranty period.

Standard warranty period is 3 years, or extended 10 years.

B.	Window Material and Workmanship

1.	Provide written guarantee against defects in material and workmanship for ___ years from the date of final shipment.

C.	Glass

1.	Provide written warranty for insulated glass units that they will be free from obstruction of vision as a result of dust or film formation on the internal glass surfaces caused by failure of the hermetic seal due to defects in material and workmanship.

2.	Warranty period shall be for 10 (ten) years.

D.	Finish

Select for Anodic Finish:  Standard warranty period is 3 years, or extended 5 years, or 10 years.

1.	Warranty period shall be for ___ years from the date of final shipment.

OR

Select for Organic Finish:  Standard warranty period is 5 years, or extended 10 years, or 15 years.

1.	Warranty period shall be for ___ years from the date of final shipment.

2.	Provide organic finish warranty based on AAMA standard 2604.

OR

Select for Organic Finish:  Standard warranty period is 5 years, or extended 10 years, or 15 years, or 20 years.

1.	Warranty period shall be for ___ years from the date of final shipment.

2.	Provide organic finish warranty based on AAMA standard 2605.

PART 2 PRODUCTS

2.01	Materials

A.	Aluminum

1.	Extruded aluminum shall be 6063-T6 alloy and temper.

A complete glass description is required for the manufacturer to supply appropriate glazing gaskets even if glass and glazing is by others.  Follow local building codes regarding tempered glass.

- Glass

- Ship open for 1” Insulated glass with a center of glass U-Factor of ___ constructed as follows:

a.	Exterior lite - ___ thick, ___ color, ___ glass (annealed, H.S. Temp), 
with a surface coating of ___ on the number ___ (1 or 2) surface.

b.	Air space of ___ inch (or argon filled).

c.	Interior lite - ___ thick, ___ color, ___ glass (annealed, H.S. Temp), 
with a surface coating of ___ on the number ___ (3 or 4) surface.

OR

1.	Ship open for monolithic glass by others

a.	Exterior lite - ___ thick, ___ color, ___ glass (annealed, H.S. Temp), 
with a surface coating of ___ on the number ___ (1 or 2) surface.

C.	Thermal Barrier

    - All exterior aluminum shall be separated from interior aluminum by a rigid, structural thermal barrier. For purposes of this specification, a structural thermal barrier is defined as a system that shall transfer shear during bending and, therefore, promote composite action between the exterior and interior extrusions.

    - Barrier material shall be poured-in-place, two-part polyurethane. A nonstructural thermal barrier is unacceptable.

Insert at discretion of specifier.

D.	Sustainable Design

    - The materials specified in this section are intended to have applicable credits toward LEED® certification. Provide documentation in accordance with USGBC’s LEED for New Construction and Major Renovation (insert one) version 2.2 or version 3 (2009), verifying that the components, processes or assemblies specified herein conform to the following requirements.

Select the credits required for the specified level of certification being sought.  Modify the criterion as needed to coincide with other LEED® standards.

- EA Credit 1: Optimize Energy Performance

- Provide area weighted  overall U-Factor in accordance with section 1.05

ii.	Provide area weighted center of glass or overall SHGC in accordance with section 1.05

- EA Credit 2: On site renewable energy

i.	Provide coordination for glazing of photovoltaic panels or other energy generating glazing.

- MR Credit 1.1 and 1.2: Building Reuse

i.	Provide windows and accessories which allow easy retrofit and minimal demolition into existing building conditions.

- MR Credit 4.1 and 4.2: Recycled Content.

- Provide aluminum framing with overall recycled content values of minimum 80%. All secondary aluminum must comply with alloy and temper requirements outlined in section 2.01. A.1.

ii.	Overall recycled content must contain the following levels of post and pre consumer recycled content.

- Post consumer content no less than 55%

- Pre consumer content no less than 22%

iii.	Submit documentation outlining recycled content percentages (post and pre consumer content separately) and the weight of aluminum in proportion to the overall assembly weight.

NOTE: Glass components are a large portion of total assembly weight and do not contribute to the recycled content value.

- MR Credit MR 5.1 and 5.2: Regional Materials (only for projects located within 500 miles of Monett Missouri)

i.	Provide assemblies that are manufactured as well as use materials that are harvested, recovered or extracted from within 500 miles of the job site.

- EQ Credit 2: Increased ventilation

- Provide operable vents with occupant-enabled hardware.

ii.	Provide operable vent quantity and sizes as shown on project drawings.

- EQ Credit 4.1 and 4.2: Low-Emitting Materials (adhesives, sealants / paints and coatings)

i.	Verify that the as shipped product does not require any on site application of adhesives, sealants, paints or coatings other than perimeter or installation sealants.

ii.	All interior “on site” applied primers, structural glazing adhesives, metal to metal sealants and cosmetic seals must meet applicable South Coast Air Quality Management District (SCAQMD) Rule #1168 VOC limits.

- EQ Credit 6.1: Controllability of Systems - Lighting

i.	Provide windows with internal occupant controlled blinds.

- EQ Credit 6.2: Controllability of Systems -  Thermal Comfort

- Provide operable vents with occupant-enabled hardware.

- Provide operable vent quantity and sizes as shown on project drawings.

- EQ Credit 8.1 and 8.2: Daylight and Views

- Provide Glass with visual light transmittance (VT) as outlined in section 2.01 E.

ii.	Provide operable vent quantity and sizes as shown on project drawings.

2.02	Fabrication

A.	General

1.	All aluminum frame extrusions shall have a minimum wall thickness of .080" (2 mm).

2.	All exposed work shall be carefully matched to produce continuity of line and design with all joints. System design shall be such that raw edges will not be visible at joints.

B.	Frame

1.	Depth of frame shall not be less than 6 1/2" (165 mm).

2.	Face dimension shall not be less than 2" (50 mm).

3.	Frame components shall be screw spline construction.

4.	Door frame shall have shear blocked horizontals between the door jambs with screw spline sidelights.

C.	Glazing

1.	All units shall be “dry glazed" with gaskets on both exterior and interior of the glass.

2.03	Finish

See separate finish specification for more details.

- Anodic

  - Finish all exposed areas of aluminum windows and components with electrolytically deposited color in accordance with Aluminum Association Designation

AA-M10-C22-___ Color shall be ___.

Available colors are clear, champagne, light bronze, medium bronze, dark bronze & black.

AA Description	Description	Arch. Class	AAMA Guide Spec.

AA-M10-C22-A41	Clear Anodized	1	611-98

AA-M10-C22-A44	Color Anodized	1	611-98

OR

- Organic

  - Finish all exposed areas of aluminum windows and components with ___. Color shall
be ___.

High performance 70% PVDF fluoropolymer Ultrapon™ and 50% PVDF fluoropolymer Ultraflur™ finishes are available in standard colors.  Reference the EFCO color brochure for assistance in color and resin selection.  Custom colors are available upon request.

AA Description	Description	AAMA Guide Spec.

AA-M12-C42-R1X	70% PVDF Ultrapon™	2605-98

AA-M12-C42-R1X	50% PVDF Ultraflur™	2604-98

PART 3 EXECUTION

3.01	Inspection

A.	Job Conditions

1.	All openings shall be prepared by others to the proper size and shall be plumb, level and in the proper location and alignment as shown on the architect's drawings.

Insert at discretion of specifier

2.	Provide for manufacturer representation to conduct pre-installation site meeting.

3.02	Installation

A.	Use only skilled tradesmen with work done in accordance with approved shop drawings and specifications.

B.	Storefront system shall be erected plumb and true, in proper alignment and relation to established lines and grades.

C.	Entrance doors shall be securely anchored in place to a straight, plumb and level condition, without distortion. Weather stripping contact and hardware movement shall be checked and final adjustments made for proper operation and performance of units.

D.	Furnish and apply sealing materials to provide a weather tight installation at all joints and intersections and at opening perimeters.

E.	Sealing materials specified shall be used in strict accordance with the manufacturer‘s printed instructions, and shall be applied only by mechanics specially trained or experienced in their use. All surfaces must be clean and free of foreign matter before applying sealing materials. Sealing compounds shall be tooled to fill the joint and provide a smooth finished surface.

3.03	Anchorage

A.	Adequately anchor to maintain positions permanently when subjected to normal thermal movement, specified building movement, and specified wind loads.

3.04	Protection and Cleaning

A.	The general contractor shall protect the aluminum materials and finish against damage from construction activities and harmful substances. The general contractor shall remove any protective coatings as directed by the architect, and shall clean the aluminum surfaces as recommended for the type of finish applied.

EFCO is a registered trademark of EFCO Corporation. Ultrapon and Ultraflur are trademarks of EFCO Corporation.

6/13/2011