- PART 1   GENERAL

- SECTION INCLUDES

- Aluminum clad wood commercial door and frame, complete with hardware, glazing, weather strip, removable grille, simulated divided lite, stationary sidelite, stationary transom, jamb extension, and standard or specified anchors, trim, and attachments.

- RELATED SECTIONS

- Section 01 33 23—Submittal Procedures:  Shop Drawings, Product Data, and Samples

- Section 01 62 00—Product Options

- Section 01 63 00—Product Substitution Procedures

- Section 01 65 00—Product Delivery

- Section 01 66 00—Product Storage and Handling Requirements

- Section 01 71 00—Examination and Preparation

- Section 01 73 00—Execution

- Section 01 74 00—Cleaning and Waste Management

- Section 01 75 00—Starting and Adjusting

- Section 01 76 00—Protecting Installed Construction

- Section 06 22 00—Millwork:  Wood trim other than furnished by door and frame manufacturer

- Section 07 92 00—Joint Sealants:  Sill sealant and perimeter caulking

- Section 08 71 00—Door Hardware:  Hardware other than furnished by door and frame manufacturer

- Section 09 90 00—Paints and Coatings:  Paint or stain other than factory applied finish

- REFERENCES

Include only those reference standards that are referred to within the text of this section.  Edit for project conditions.

- WDMA I.S.4:  Industry Standard for Water Repellent Preservative Treatment for Millwork.

- Sealed Insulating Glass Manufactures Association / Insulating Glass Certification Council (SIGMA / IGCC).

- American Architectural Manufacturers Association (AAMA):  2605:  Voluntary Specification for High Performance Organic Coatings on Architectural Extrusions and Panels.

- National Fenestration Rating Council (NFRC): 101:  Procedure for Determining Fenestration Product Thermal Properties.

- SUBMITTALS

- Shop Drawings:  Submit shop drawings under provisions of Section 01 33 23.

- Product Data:  Submit catalog data under provisions of Section 01 33 23.

- Samples:

- Submit corner section under provisions of Section 01 33 23.

- Include glazing system, quality of construction, and specified finish.

- Quality Control Submittals:  Certificates:  Submit manufacturers certifications indicating compliance with specified performance and design requirements under provisions of Section 01 33 23.

- DELIVERY

- Comply with provisions of Section 01 65 00.

- Deliver in original packaging and protect from weather.

- QUALITY ASSURANCE

Include the following article only for projects including egress requirements.  Edit applicable building code.

- Regulatory Requirements:  Emergency Egress or Rescue:  Comply with requirements for sleeping units of [IBC International Building Code] [BOCA Basic Building Code] [Southern Building Code] [Uniform Building Code] [_________________].

- STORAGE AND HANDLING

- Prime or seal wood surfaces, including surface to be concealed by wall construction, if more than thirty (30) days will expire between delivery and installation.

- Store door panels flat on a level surface in a clean and dry storage area under provisions of Section 01 66 00.  Seal unfinished top and bottom edges of door panels if door panels are stored at the job site more than one (1) week.

- Condition doors to local average humidity before hanging.

- WARRANTY

The following limited warranty is subject to conditions and exclusions.  There are certain conditions or applications over which Marvin has no control.  Defects or problems as a result of such conditions or applications are not the responsibility of Marvin.  For a more complete description of the Marvin limited warranty, refer to the Marvin Windows and Doors Ten-Year Limited Warranty.

- Doors shall be warranted to be free from defects in manufacturing, materials, and workmanship for a period of ten (10) years from purchase date.

- Insulating glass shall be warranted against visible obstruction through the glass caused by a failure of the insulating glass air seal for a period of twenty (20) years from the date of orginal purchase.

- Please see separate manufacturer warrenties for Commercial Door hardware components, sills and steel frames.

- PART 2   PRODUCTS

- MANUFACTURED UNITS

- Description:  Factory assembled aluminum clad Commercial Door, as manufactured by Marvin Windows and Doors, Ripley, Tennessee.

- FRAME DESCRIPTION

- Finger jointed, edge-glued pine core with clear pine veneer; finger jointed Douglas fir core with clear Douglas fir veneer, finger jointed, edge-glued white oak core with clear white oak veneer; finger jointed, edge-glued cherry core with clear cherry veneer; finger jointed, edge-glued mahogany core with clear mahogany veneer; finger jointed, edge-glued vertical grain Douglas fir core with clear vertical grain Douglas fir veneer.

- Kiln dried to a moisture content no greater than twelve (12) percent at the time of fabrication.

- Water repellent, preservative treated in accordance with WDMA I.S.4.

- Frame width: 4-9/16 inches (116 mm); 6-9/16 inches (167 mm).

- Frame thickness: 1-1/16 inches (27 mm).

- Exterior extruded aluminum clad 0.050 inch (1.3 mm) thick.

- Low profile sill (No sill – sill supplied and applied by others. Jambs extended 7/8 inch (22 mm) beyond panel bottom).  (Steel frame with unequal rabbet 16 gauge:  4 ¾” (121), 5 ¾”(146), 6 ¾”(171) or 7 ¾” (197) frame width.  Specify wood stud, wire masonry anchor or existing opening anchor.).

  - PANEL DESCRIPTION

- Stiles:  finger jointed, edge-glued LVL. Rails:  finger jointed, edge-glued pine,  Douglas fir, white oak, cherry, mahogany, vertical grain Douglas fir cores with clear pine, white oak, cherry, mahogany, vertical grain Douglas fir veneer.

- 1.	Kiln dried to a moisture content no greater than twelve (12) percent at time of fabrication.

- 2.	Water repellent, preservative treated in accordance with WDMA I.S.4.

- B.	Stiles contain laminated veneer lumber (LVL) core, solid wood top ad bottom rail, with clear pine,Douglas fir,  white oak, cherry, mahogany, vertical grain Douglas fir veneers.

- C.	Composite panel thickness:  1-3/4 inches (44mm); 2 ¼ inches (57mm).

- D.	Exterior extruded aluminum clad 0.055 inch (1.4 mm) thick.

- E.	Top rail width:  1 ¾ inch panel 6 inches (152 mm); 2 ¼ inch panel 8 1/8 inches (206 mm).

- F.	Stile width:  6 inches (152 mm).

- G.	Bottom rail height:  11-3/8 inches (289 mm).

- H.	Panel corners glued and fastened with 5/8 X 4 inch (16 mm X 102 mm) fluted hardwood dowels.  Removable interior vinyl glazing stops with clear wood covers; 1 ¾ inch panel - no visible fastener holes (2 ¼ inch panel – nailed on glazing stops).

- 2.4	GLAZING

- A.	Select quality complying with ASTM C 1036.  Comply with 16 CFR 1201 Safety Standard for Architectural Glazing Materials.  Tempered insulating glass IGMA/IGCC certified to performance level CBA when tested in accordance with ASTM E 774.

- Glazing Method:  Tempered Insulating Glass (Altitude Adjusted).

- Glass Type: Clear; Bronze; Gray; Reflective Bronze; Low E II—Argon Gas; Obscure; Laminated.

- D.	Glazing Seal:  Silicone bedding, exterior.

- 2.5	FINISH

- A.	Exterior:  Aluminum clad.  Fluoropolymer modified acrylic topcoat applied over primer.  Meets or exceeds AAMA 2605 requirements.

- 1.	Standard Color:  Stone White; Bahama Brown; Bronze; Pebble Gray; Evergreen.

- 2.	Select Color:  Sierra White; Coconut Cream; French Vanilla; Cashmere; Desert Beige; Cumulus Gray; Cadet       Gray; Ebony; Arctic White; Cascade Blue; Cobalt Blue; Hampton Sage; Sherwood Green; Wineberry; Custom color – contact your Marvin representative.

- Interior:  Treated bare wood; Latex prime coat, white – available for pine only.

  - HARDWARE

- A.	Hinges:  4 ½” X 4 ½” square corner ball bearing hinges. Finish:  Satin Chrome; Bronze; Brass; Stainless Steel.

- B.	Locking System:  No lock/no bore (Von Duprin 98 L) ( Von Dupirn 98 EO) Schlage L series entrance) Schlage L series corridor) (head and foot bolt on inactive panel).

  - WEATHER STRIP

- Head jamb and side jambs to have two sets of bulb weather strip, locking stiles have pile weather strip maintaining contact with door panels.

  - JAMB EXTENSION

- Factory installed (loose), for wall thickness indicated or required.

- B.	Finish:  Match interior frame wood species and finish.

  - REMOVABLE GRILLES

- 3/4 X 15/32 inch (19 mm X 12 mm); 1-1/8 X 15/32 inch (29 mm X 12 mm) Pine only.

- Pattern:  Rectangular; Custom lite layout.

- Finish:  Match interior panel finish.

  - SIMULATED DIVIDED LITES (SDL)

- 7/8 inch (22 mm) wide; 1 1/8 inch (29 mm) wide (Internal spacer bars).

- Exterior muntins:  Extruded aluminum 0.055 inch (1.4 mm) thick.

- Interior muntins:  Pine; Douglas fir;  white oak; cherry; mahogany; vertical grain Douglas fir. SDL adhered to glass with closed-cell copolymer acrylic foam tape.

- Pattern:  Rectangular; Custom lite layout.

- Finish:  Match panel finish.

  - GRILLES-BETWEEN-THE-GLASS (GBG)

- 11/16” (17 mm) white contoured aluminum bar. Optional flat aluminum spacer bar, contact your Marvin representative.

- Various types of sills are available.

  - ACCESSORIES AND TRIM

- Installation and Hardware Accessories:

- Factory installed vinyl nailing fin/drip cap.

- Installation brackets:  6-3/8 inches (162 mm); 9-3/8 inches (238 mm); 15-3/8 inches (390 mm).

- Masonry brackets:  6 inches (152 mm); 10 inches (254 mm).

- Aluminum Extrusions:

- Profile:  Brick mould casing; Flat casing; Frame expander; Jamb extender; Mullion cover; Mullion expander as indicated on drawings.

- 2.	Finish:  Fluoropolymer modified acrylic topcoat applied over primer.  Meets or exceeds AAMA 2605 requirements.  a.	Standard Color:  Stone White; Bahama Brown; Bronze; Pebble Gray; Evergreen.

- b.	Select Color:  Sierra White; Coconut Cream; French Vanilla; Cashmere; Desert Beige; Cumulus Gray; Cadet Gray; Ebony; Arctic White; Cascade Blue; Cobalt Blue; Hampton Sage; Sherwood Green; Wineberry; Custom color – contact your Marvin representative.

- PART 3   EXECUTION

- EXAMINATION

- Verification of Conditions:  Before Installation, verify openings are plumb, square, and of proper dimension as required in Section 01 71 00.

- Acceptance of Conditions:  Beginning of installation confirms acceptance of existing conditions.

- INSTALLATION

- Comply with Section 01 73 00.

- Assemble and install doors and frames according to manufacturer’s instructions and reviewed shop drawings.

- Install frames and stationary panels as required.

- Install accessory items as required.

- Use finish nails to apply wood trim and mouldings.

- STARTING AND ADJUSTING

- Adjust door to work freely with hardware functioning properly.

- CLEANING

- Remove visible labels and adhesive residue according to manufacture’s instructions.

- Leave doors and glass in a clean condition.  Final cleaning as required in Section 01 74 00.

- PROTECTING INSTALLED CONSTRUCTION

- Comply with Section 01 76 00.

- Cover doors and frames during painting or other construction operations that may cause damage.

END OF SECTION