*************************************************************************** 
NAVFAC                                                 PTS-E20 (September 2022)
                                             ------------------------------
Preparing Activity: NAVFAC                  SUPERSEDING PTS-E20 (December 2018)

PERFORMANCE TECHNICAL SPECIFICATION
***************************************************************************

SECTION E20

FURNISHINGS
09/22

*************************************************************************** 
NOTE:  This section is intended to be used as a guide and contains requirements that are common to many different types of facilities; however, not all requirements and equipment items will be applicable to all projects.  In addition, there may be special requirements for a particular project that are not addressed at all.  The RFP preparer may have to incorporate additional information to address these special requirements in this PTS and corresponding Part 3 ESR.  If the RFP preparer chooses to delete building elements that are not required for the project, do not change the remaining Uniformat paragraph designations (example - A102001). Uniformat designations are unique to the products they are assigned to. However, the subparagraph numerical extensions (example – 1.2 or a,b,c) of the Uniformat designations may change if subparagraphs are deleted.

This guide specification is formatted utilizing Uniformat II, an industry recognized standard, ASTM E 1557.  When the RFP preparer chooses to add a paragraph that does not apply to an existing building element already included in the specification, refer to the Uniformat/WBS located on the NAVFAC Design-Build Website for a listing of Uniformat II designations and definitions.

NOTE:  The RFP preparer may view or hide the criteria notes in this PTS section by modifying the WORD preferences for "Hidden text".  To view the criteria notes, choose "File" then "Option".  Click "Display" then check the "Hidden text" box under "Always show these formatting marks on the screen".  In the same section, check the box for "Print hidden text" under "Printing options" to print the criteria notes.
***************************************************************************

*************************************************************************** 
NOTE:  The Table of Contents is intended for navigation purposes only for the RFP writer and should not show up in the printed document.

TABLE OF CONTENTS

E20  GENERAL
E20  1.1  DESIGN GUIDANCE
E20  1.2  GENERAL REQUIREMENTS
E20  1.3  DESIGN SUBMITTALS
E20  1.4  CONSTRUCTION SUBMITTALS
E2010  FIXED FURNISHINGS
E201001  FIXED ARTWORK
E201002  WINDOW TREATMENTS
E201002  1.1  BLINDS
E201002  1.2  SHADES
E201002  1.3  DRAPERIES AND HARDWARE
E201003  SEATING (FIXED)
E201003  1.1  AUDITORIUM, LECTURE AND CLASSROOM SEATING
E201003  1.2  FIXED TABLES FOR AUDITORIUM, LECTURE AND CLASSROOMS
E2020  MOVABLE FURNISHINGS
E2020  1.1  FF&E PACKAGE
E2020  1.2  PURCHASE AND INSTALLATION
E2020  1.3  BEST VALUE DETERMINATION
E202001  MOVABLE ART WORK
E202002  MODULAR PREFABRICATED FURNITURE
E202002  1.1  FURNITURE SYSTEMS
E202002  1.2  MODULAR FREESTANDING FURNITURE AND WORKSTATIONS
E202003  FREESTANDING FURNITURE
E202003  1.1  SEATING
E202003  1.2  STORAGE AND FILING
E202003  1.3  CHILD DEVELOPMENT CENTERS
E202003  1.4  TRAINING/CONFERENCE ROOM FURNISHINGS
E202003  1.5  DINING ROOM FURNISHINGS
E202004  RUGS & ACCESSORIES
E202004  1.1  RUGS
E202004  1.2  LAMPS
E202004  1.3  INTERIOR LANDSCAPING
E202004  1.4  OTHER DECORATIVE ITEMS
E202090  OTHER MOVABLE FURNISHINGS
E202090  1.1  MISCELLANEOUS ITEMS
***************************************************************************

E20 GENERAL

RFP Part 3 including the Engineering System Requirements (ESR) provide project specific requirements.  The RFP Part 4, Performance Technical Sections (PTS) provide generalized technical requirements that apply to multiple facility types and include more requirements than are applicable to any one project. Therefore, only the RFP Part 4 requirements that apply to the project and further define the RFP Part 3 project specific requirements are required.

E20 1.1 DESIGN GUIDANCE

Provide the design and installation in accordance with the following references.  This Performance Technical Specification (PTS) adds clarification to the fundamental requirements contained in the following Government Standards.  The general requirements of this PTS section are located in PTS Section Z10, General Performance Technical Specification.

Industry standards, codes, and Government standards referenced in the section text that are not found in the Unified Master Reference List (UMRL)  in the Federal Facility Criteria (FFC)  at the Whole Building Design Guide (WBDG)  website, are listed below for basic designation identification.  Comply with the required and advisory portions of the current edition of the standard at the time of contract award:

*************************************************************************** 
NOTE: Edit the following list of industry and Government standards to coordinate with the section text additions and deletions.  List all industry and Government standards in the following two paragraphs that are indicated in the section text that are not listed in the UMRL.
***************************************************************************

E20 1.1.1 Industry Standards and Codes

E20 1.1.2 Government Standards

UNIFIED FACILITIES CRITERIA (UFC)

| UFC 1-200-01 | DoD Building Code (General Building Requirements)(A reference in this PTS section to UFC 1-200-01 requires compliance with the Tri-Service Core UFCs that are listed therein, which includes the following significant UFC(s):UFC 3-101-01, ArchitectureUFC 3-120-10, Interior Design) |
| --- | --- |
| UFC 1-200-02 | High Performance and Sustainable Building Requirements |

E20 1.2 GENERAL REQUIREMENTS

Provide the services of an Interior Designer, certified by the National Council for Interior Design Qualification (NCIDQ), or a state and/or jurisdiction Certified, Registered, or Licensed Interior Designer prepare both the     Furniture, Fixtures, & Equipment (FF&E)  and the     Structural Interior Design (SID) Package, attend and participate, in entirety, all kick-off meetings, design meetings, to include, but not limited to, design charettes, concept design workshops, and review meetings to develop the building design, floor plan, and the FF&E package.  Provide the services of equipment specialists to specify the audiovisual, shop, or specialty equipment.  The Interior Designer and any specialists must not be affiliated with any furniture dealership/vendor or manufacturer. The Government Interior Designer reserves the right to approve/disapprove the qualifications of the Contractor's Interior Designer.

Systems furnishings installers must be the systems furniture manufacturer's approved dealer of record.  In addition, installation dealers must be located within a 100 mile radius of the project site unless approved by the government Interior Designer.

*************************************************************************** 
NOTE: Choose the following bracketed paragraph for renovation projects, if necessary.
***************************************************************************

[Re-purpose/ recycle existing furniture if not relocated by the government.  Provide verification that the existing furniture was not disposed of at the landfill.]

E20 1.3 DESIGN SUBMITTALS

Provide design submittals in accordance with Z10, General Performance Technical Specifications, Part 2 Section 01 33 10.05 20, Design Submittal Procedures,    Facilities Criteria (FC) 1-300-09N, Navy and Marine Corps Design Procedures, and UFC 3-120-10, Interior Design.

*************************************************************************** 
NOTE:  NAVFAC has made every effort to use commercial standards in the PTS sections. This PTS section is designed to only use commercial standards. If project requirements dictate the use of a UFGS sections as a standard, add a paragraph here listing the required UFGS section. State in the paragraph that the DOR must edit this UFGS section in accordance with PTS Z10 and submit it as a part of the design submittal.
***************************************************************************

E20 1.4 CONSTRUCTION SUBMITTALS

Submit construction submittals in accordance with PTS Section Z10, General Performance Technical Specifications.  In addition to the Z10 requirements, the Designer of Record (DOR) must approve the following construction submittals as a minimum:

Art work and FF&E Package

E2010 FIXED FURNISHINGS

E201001 FIXED ARTWORK

*************************************************************************** 
NOTE:  Artwork of this type could include a wall mosaic, painted murals, banners or graphic images incorporated into flooring materials.  These items would be decorative in nature and would require custom designs for the respective interior finish.
***************************************************************************

E201002 WINDOW TREATMENTS

Provide interior window coverings, associated hardware and controls at each exterior window and at any interior view window where privacy may be required. Refer to the Project Program for size, pattern and style of window treatments.  At a minimum, functional window coverings such as blinds or solar shades are required on all projects.

E201002 1.1 BLINDS

Venetian blinds must be one-inch horizontal room-darkening commercial grade aluminum at a minimum thickness of 0.2mm (.008") (after coating) with a minimum of 45.72 slats per meter (15 slats per foot). Provide blinds at each exterior window and at any interior window where privacy may be required.  Provide and install one full size sample for review and approval.

E201002 1.2 SHADES

Provide energy efficient solar shading systems for exterior windows.  The system must maintain visibility while reducing glare, solar heat gain during the summer and heat loss during the winter.  Openness configuration must be no more than 5% for most areas.  The system fabrics and components must be dimensionally stable and must be manufactured to withstand fading, fire, mildew, and soiling.

E201002 1.3 DRAPERIES AND HARDWARE

Draperies must be manufactured under     General Services Administration (GSA) contract as part of FF&E.  Drapery can be accordion-fold, ripple-fold, or pinch-pleat with associated hardware suitable for the project requirements. All draperies must meet the     National Fire Protection Association (NFPA) Code 701 California Fire Marshall’s flame retardant requirements.

E201003 SEATING (FIXED)

E201003 1.1 AUDITORIUM, LECTURE AND CLASSROOM SEATING

The system must permit the standards to be installed on radial lines from a common center for which concentric circles are determined with each row of units utilizing common middle standards.  Standards in each row must be placed laterally so the aisle-end standards will be in alignment as indicated on seating layout drawing.  The angle of inclination of backs must be adjusted for variations in sightlines.  Mechanical attachment of components must be of sufficient flexibility so that when permanently assembled they will compensate for the changing dimensions laterally between standards caused by convergence toward the center.  Seat and back attachments must absorb inaccuracies in lateral spacing of standards at point of attachment caused by unevenness of floor.  Varying lateral dimensions of backs and seats must be in accordance with approved seating layout.  Minimum width of seating unit must be 20 inches and may be used only to complete a specific row dimension.

E201003 1.2 FIXED TABLES FOR AUDITORIUM, LECTURE AND CLASSROOMS

Provide worksurfaces mounted to     tubular steel, sheet steel, or cast iron floor standards.  The standards must be formed to fit the floor incline so that the standards will be vertical.  The feet must be formed to eliminate tripping hazards and have holes for bolt attachment to the floor.  Provide riser standards, cantilevered standards and aisle and end standards as required.  Provide communications, data and power routing as required.  Provide a high-pressure plastic laminate over medium density particleboard for the worksurfaces with coordinating vinyl or resin edge detail.

E2020 MOVABLE FURNISHINGS

Furnishings, Fixtures, and Equipment (FF&E) must include furniture, shop equipment, audiovisual equipment, and specialty equipment. Weapon racks, drying cages, and lockers are not considered FF&E.   FF&E must be fully integrated with the building systems and finishes.  FF&E may also include specialty items for which the customer activity must be responsible for specifying.

Design and provide as required FF&E for all areas as developed during client programming.  Design an FF&E package and prepare supporting plans and procurement data in accordance with the general interior design requirements in UFC 3-120-10.

*************************************************************************** 
NOTE:  The E20 Part 3 Engineering Systems Requirements (ESR) should clarify if the contract requirements include the design, specification and purchase of the FF&E in the base contract.  The FF&E budget amount from the 1391 should also be included in Part 3
***************************************************************************

E2020 1.1 FF&E PACKAGE

FF&E Package: Design and provide a fully usable and complete facility to include a FF&E movable furnishings package from Government supply sources according to Federal Acquisition Regulations.  The FF&E will include, but not limited to, systems and modular furniture, training and conference furniture, seating, tables, artwork, decorative window covering, specialty furniture and equipment, dormitory room furnishings, and accessories.     Naval Supply Systems Command (NAVSUP) Blanket Purchase Agreements (BPA) must be used whenever possible.  The government will provide separate funding for the FF&E package.  Construction funds will not be used.  The FF&E Package must include shipping, freight, handling, installation and the     Handling and Administration Rate (HAR) percentage as applied to the final FF&E total cost.

E2020 1.1.1 Authorization

The government will provide separate funding for procurement of the FF&E package.  Upon receipt of required funding, the Contractor must be authorized by the Contracting Officer as a planned line item modification to the contract/task order to procure all FF&E using predominately negotiated Federal contracts.  The amount of the modification will be the actual cost of these items from the Federal price schedules or NAVSUP BPAs, including any freight and installation charges from the furniture supplier as well as the Contractor's FF&E Handling and Administration Rate (HAR).  The HAR includes all of the prime Contractor's effort related to storage, coordination, handling, administration of subcontractors, and all other associated costs and profit for the procurement of FF&E.  The prime Contractor will propose in the contract/task order solicitation the FF&E HAR.  The Contractor's proposed HAR may not exceed 5% of the total FF&E costs, as noted on the bid schedule. No other charges, expenses, fees, or markups will be authorized.

The government Interior Designer will approve the final FF&E submittal. The FF&E package will be presented to the Contracting Officer and the Contractor must provide the FF&E exactly as specified and approved.

The Contractor will receive a letter of authorization from the Contracting Officer citing the name of the furniture dealer and other information to use when accessing the government supply sources.

E2020 1.2 PURCHASE AND INSTALLATION

Coordinate the building completion date with the installation dealer specified in the FF&E Package. The Contractor or Contractor's representative is responsible for the following:  issuing purchase orders, receiving acknowledgements, sending copies of purchase orders to the installation dealer(s) specified in the FF&E package, and providing necessary deposits to furniture manufacturers.

The FF&E installation dealer(s) is responsible for the following: Receiving and installing all FF&E specified in the FF&E package, coordinating delivery and installation with the Contractor, inspecting for damage, providing delivery receipts to the Contractor, filing necessary freight claims, hanging artwork, bulletin boards, etc., removing packaging material, cleaning up the site upon completion, and adhering to Contractor's safety requirements.

E2020 1.2.1 Use of GSA Schedules and Blanket Purchase Agreements (BPAs)

The prime Contractor or FF&E dealer will be authorized to purchase supplies or services from the Navy Furniture BPAs for FF&E requirements, under the terms of the contract.  The Contractor will receive a letter of authorization from the Contracting Officer citing the name of the furniture dealer and other information to use when accessing the government supply sources or BPAs.

E2020 1.2.2 Deposits

The Contractor must anticipate providing a deposit of between 30% to 50% of the furniture costs when placing their order.

Manufacturer price increases must be anticipated.  Recommend ordering FF&E product once funds are received to avoid incurring additional costs.  Delayed production and delivery dates can be noted at the time of order placement to coincide with building completion dates.  Any costs incurred due to manufacturer price increases will be the burden of the Contractor.

E2020 1.2.3 Davis Bacon Wages

Davis Bacon wages do not apply to the FF&E installer from the government supply sources.  The workforce for the FF&E installation and delivery must be separate and distinct from the labor workforce performing under the construction contract.

E2020 1.2.4 Sales Tax

Exemptions for certain State or Local taxes may be available to the Contractor and/or its subcontractors.  The Contractor must take maximum advantage of all exemptions, including obtaining a resale permit, from State and Local taxation authorities whether available to it directly or available to the Contractor based on an exemption afforded the government. The responsibility for paying applicable taxes rests with the contractor. State and local taxes applicable to the FF&E line will be included with the subcontractor's quote, if applicable.

E2020 1.2.5 Bonds

FF&E line item is not considered construction and the prime Contractor will not be required to secure any additional bond for the award of the FF&E line item unless otherwise indicated in the RFP.  If any additional bond is required for the FF&E line item it is to be included in the prime Contctor's FF&E HAR.

E2020 1.2.6 Unique Item Identification (IUID) and Validation

Unique item identification and valuation is a system of marking and valuing items delivered to DoD that enhances logistics, contracting, and financial business transactions.  The IUID policy is mandatory for all DoD contracts that require the delivery of items.  An item is a single article or a single unit formed by a grouping of subassemblies, components or constituent parts.  Provide DoD Unique item identification, valuation and delivery of data for all required FF&E items for which the government's unit acquisition cost is $5,000 or more.

E2020 1.2.7 Buy American Act and Trade Agreement Act

All supplies under the FF&E line item are subject to the Buy American Act and Trade Agreement Act (TAA). The GSA contracts and NAVSUP Blanket Purchase Agreements are required to comply with the Buy American Act and TAA.

E2020 1.2.8 Small Business Requirements

The FF&E is subject to the Contractor's Small Business Goals however the government requires the furniture be purchased from NAVSUP Blanket Purchase Agreements (BPA). Most manufacturers on the Office Furniture BPA are large business and most manufacturers on the Dorm and Quarters BPA are small business.  Installation dealers are small business.  Under the terms of the BPA, the FF&E must be ordered directly through the GSA manufacturer.  Using pass-through companies to achieve Small Business Goals will not provide the Contractor credit unless they manufacturer 20% or provide 50% of the service purchased.  The government will not incur additional costs to use small business.

E2020 1.2.9 Installation

The FF&E package includes the installation of all furniture and furnishings as specified in the FF&E package.  The installation dealer specified in the FF&E package will receive, store, if required, transport to the project site, off load, inside deliver, unpack, assemble, place/install, clean, if required, and dispose of all the trash for all furniture and furnishings. The Contractor's Interior Designer will be responsible for specifying installation services and warehousing, as required, for all collateral equipment.  It is the Contractor's responsibility to coordinate the building completion, occupancy, and furniture installation dates with the installation dealer specified in the FF&E package.  Any costs associated with storing or delaying furniture shipments is the responsibility of the Contractor.

E2020 1.2.10 Installation Warranty

Install all movable furnishings in accordance with the manufacturer’s instructions and warranty requirements.  All movable furnishings must be level and aligned and all doors, drawers and accessories must be level and aligned to open, close and otherwise operate smoothly and securely.  All systems furniture must be installed by the systems furniture manufacturer’s dealer of record and not the General Contractor. Repair, to the customer’s satisfaction, any/all damage to any facility finish that is a result of the furniture installation and correct all punch list items for the furniture/furnishings.

E2020 1.2.11 Ordering Documentation

Provide two copies of all ordering documentation to the Contracting Officer including Factory Order number (FO) and warranty information.

E2020 1.2.12 Post Award Changes

After award of the FF&E line item modification, any request to change the FF&E items must be submitted on the Contracting Officer.  The FF&E  modification has been accepted, priced, and negotiated based on specific line items as detailed in the final package.  Those items have been agreed to considering color, specific type and quality of material, price, sustainability, life cycle, and dealership service.  The Government will expect and require the Contractor to provide exactly those items.  Should changes become necessary, careful consideration is required to ensure that equivalent quality, price and other aspects of the item are maintained.  Otherwise, price adjustments must be negotiated.  The Contracting Officer will obtain approval from the Government Interior Designer/Collateral Equipment Manager in consultation with the client for any changes to the FF&E.

Post award FF&E manufacturer's price increases are the responsibility of the Contractor and will not be transferred to the government. Recommend ordering FF&E product once funds are received to avoid incurring additional costs.  Delayed production and delivery dates can be noted at the time of order placement to coincide with building completion dates.

E2020 1.3 BEST VALUE DETERMINATION

A best value determination is required by     Federal Acquisition Regulation (FAR) 8.404 when placing orders against Federal Supply Schedules for the selection of furniture and furnishings. A     Best Value Determination (BVD) must also be provided for FF&E installation services.  Best Value is defined in FAR 2.101 as ensuring that the order to be placed under a Federal Supply Schedule results in the lowest overall cost alternative (considering price, special features, administrative costs and client's needs) to meet the government's needs.

The Contractor's Interior Designer is responsible for the following written BVD justifications:

$3,000 or less:  For any procurement in the FF&E package with a value of $3,000 or less, the Interior Designer may utilize any BPA holder.  If the BPA holders cannot supply the item, then any other manufacturer may be utilized.

Greater than $3,000 and $150,000 or less:  for any procurement in the FF&E package with a value greater than $3,000 and $150,000 or less, the Contractor's Interior Designer must always review pricing from at least three manufacturers as well as UNICOR.  In addition to the review of published list prices, the Contractor's Interior Designer must confirm the pricing with the vendor.  Manufacturer's quotes are NOT required.  The BVD form must be completed and submitted for all FF&E procurements greater than $3,000 and $150,000 or less.

Greater than $150,000:  The Contractor's Interior Designer must solicit proposals from all BPA holders under the applicable group for FF&E procurements greater than $150,000.  UNICOR must always be solicited.  The Contractor's Interior Designer must develop performance criteria and project requirements based on a generic design for the BPA holders and UNICOR to develop a price and performance proposal. The BVD form must be completed and submitted for all FF&E procurements greater than $150,000 and manufacturer's quotes and a summary of all proposals must be attached.

Federal Prison Industries (UNICOR) must be considered as part of all BVDs. This must be done by sending an email with the requirements and evaluation criteria if they are not comparable in one or more areas of price, quality, and time of delivery, the designer can specify product under NAVSUP BPA or GSA schedule.

The best value determination must address issues such as space planning; human factors data related to anthropometrics (reach, clearance, adjustability), space, and acoustics; ergonomics; product quality (including construction and materials); sustainability features, product warranties; history of the product and/or manufacturer; ability to service products through dealers or others within a certain geographical range of the project; price (including freight); aesthetics; appropriateness; and lighting, power and telecommunications systems management and/or coordination as related to the facility (when applicable); and other project specific factors as identified and/or required.  Emphasis must be to create a fully integrated design solution by providing quality products to meet the functional needs of the customer. Customer preferences must be considered. The focus must be on the best overall value. Use the GSA Best Value Determination forms provided in Part 6 of this RFP as guidelines for information to be provided.

E202001 MOVABLE ART WORK

Provide artwork for wall installation as part of the FF&E Submittal according to the project program.  Installation of artwork to be completed by installation dealer specified in the approved FF&E package. Type of artwork to be determined by client requirements and budget as described in the Project Program for the project. Install framed artwork 63 inches (1600 mm) on center above finished floor.  Include security mounting hardware as required.

E202002 MODULAR PREFABRICATED FURNITURE

E202002 1.1 FURNITURE SYSTEMS

Provide products that meet the NAVFAC performance specifications for systems furniture. The Government Interior Designer must approve any other systems furniture manufacturer.  The typical workstation must maximize each allocated space with worksurfaces and overhead closed storage with a surface to accommodate a Government provided computer.  An attached articulated keyboard/mouse tray must be selected or provided. Provide a monitor lift if required by the project program.

Powered raceways that will accommodate data and voice wire management must be completely coordinated with all facility systems.  The Contractor's Interior Designer must ensure the coordination of all electrical/data and furniture locations. Use of power poles will not be permitted to power FF&E. Provide and coordinate all telecommunication receptacles and outlet requirements (i.e. RJ 11/45 receptacles and cover plates) with the Contractor's Interior Designer and the systems furniture installer.  Hardwire all pre-wired furniture with the building systems, and coordinate all     Information technology (IT) and telephone connections.

E202002 1.2 MODULAR FREESTANDING FURNITURE AND WORKSTATIONS

Provide products that meet the NAVFAC performance specifications for modular freestanding furniture including wood. Provide modular furniture with electrical/data cable trays and grommet holes for private offices and smaller work areas.  An attached articulated keyboard/mouse tray (and monitor lift if required by the project program) must be selected.  Provide wood surfaces as appropriate.  Include accommodation for a Government provided computer and printer.

E202003 FREESTANDING FURNITURE

E202003 1.1 SEATING

E202003 1.1.1 Task Seating

Provide task seating that is fully ergonomic and coordinated by finish and scale to the workstation.  Seating specifications to include: adjustable arms, back, height, and seat pan; 5 star base, appropriate castors for floor surface, lumbar support and availability in a minimum of two (2) sizes. Task seating can be from the same manufacturer as the systems or major furniture supplier or other seating manufacturer as approved by the government Interior Designer, provided it is determined to be a BPA "Best Value".

E202003 1.1.2 Lounge, Reception and Guest Seating

Provide lounge, reception and guest seating with upholsteries consistent with the Project Program.  Seating must be easily reupholstered or have removable covers.

E202003 1.1.3 Training Room Seating

Provide seating with passive ergonomic features in that the seat and back offers some synchronized movement to allow the person seated to change positions.  The support can be legs, sled base or single pedestal with 5 star base and casters as determined by user requirements. Provide high density stack chairs or nesting chairs as required.  Provide attached, storable tablet arm as required.

E202003 1.2 STORAGE AND FILING

Provide freestanding storage units, file cabinets and file safes to accommodate the specific and unique storage requirements of the user.  Coordinate closely with storage provided in PTS Section E10, Equipment, for high density filing.

E202003 1.3 CHILD DEVELOPMENT CENTERS

Provide all child care and classroom furnishings such as cribs, rocking chairs, feeding chairs, tables, seating, toy storage, book display storage, activity centers, freestanding floor activity toys, and any other required furnishings.  Outdoor structural play equipment may be required.  The cribs must be of stainless steel construction.  The chairs and tables must be appropriately sized for each age group.

E202003 1.4 TRAINING/CONFERENCE ROOM FURNISHINGS

E202003 1.4.1 Tables

Provide single or multi-person worksurfaces or tables as determined from user requirements. Where computers are used, provide tables with wire management capability or pre-wired tables to accommodate data/telecommunications requirements.  Powered raceways that will accommodate data and voice wire management must be completely coordinated with all facility systems.  The Contractor's Interior Designer must ensure the coordination of all electrical/data and furniture locations.  Provide and coordinate all telecommunication receptacles and outlet requirements, hardwire all pre-wired furniture with the building systems and coordinate all IT and telephone connections.

E202003 1.4.2 Movable Presentation Furnishings

Provide audio-visual carts, lecterns, multi-media presentation cabinets to accommodate power/data requirements.

E202003 1.5 DINING ROOM FURNISHINGS

E202003 1.5.1 Cafeteria, Dining Hall Furnishings

Provide stackable seating and associated dollies for transport and storage, tables with folding or fixed legs or flip tops, and associated mobile carts for transport and storage, and trash and recycling containers.

E202003 1.5.2 Executive Dining Furnishings

Provide premium quality dining furnishings to include tables, chairs, buffet and serving units, hutches and cabinets as require by the Project Program.

E202004 RUGS & ACCESSORIES

E202004 1.1 RUGS

Provide Area or Accent rugs. Refer to the Project Program for requirements.

E202004 1.2 LAMPS

Provide decorative lamps.  Refer to the Project Program for requirements.

E202004 1.3 INTERIOR LANDSCAPING

Provide artificial plants of a variety of sizes and 'species' to provide interior landscaping, including planters or containers for artificial plants to coordinate with the interior finishes and furnishings of the project.

E202004 1.4 OTHER DECORATIVE ITEMS

Refer to the Project Program for requirements.

E202090 OTHER MOVABLE FURNISHINGS

E202090 1.1 MISCELLANEOUS ITEMS

E202090 1.1.1 Containers

Provide waste receptacles, recycling containers, and ash urns as required.

E202090 1.1.2 Accessories

Provide clocks, wall mounted or freestanding literature and coat racks, and mirrors as required.

E202090 1.1.3 Small Equipment

Provide small appliances such as coffee pots, microwaves, refrigerators, washers, dryers, and icemakers, as required.

E202090 1.1.4 Special Equipment

Provide mailroom work and storage tables, mail sorter units, carts and equipment tables as required.

-- End of Section --