SECTION 01505 - EXTERIOR REGULATED AREAS - LEAD-BASED PAINT

PART 1 - GENERAL

1.1	RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 1 Specification Sections, apply to this Section.

1.2	RELATED WORK SPECIFIED ELSEWHERE:

A.	Temporary Facilities:  Is specified in Section 01504.

B.	Worker Protection:  is specified in Section 01555.

C.	Respiratory Protection: is specified in Section 01556.

1.3	DESCRIPTION OF WORK:

A.	Work of this section consists of preparing a Regulated Area for exterior work of the following specification sections.

1.	Section 02066	Removal of Lead Contaminated soil

2.	Section 05582	Sheet Metal Enclosure - Lead-Based Paint

3.	Section 06107	Exterior Siding & Enclosure - Lead-Based Paint

4.	Section 06402	Carpentry and Trim Work - Lead-Based Paint

5.	Section 06403	Repair of Lead-Based Painted Substrates

6.	Section 09940	Encapsulation - Lead-Based Paint

7.	Section 09951	Chemical Stripping of Lead-Based Paint

8.	Section 09952	Mechanical Removal of Lead-Based Paint

9.	Section 09953	Surface Preparation - Paint Stabilization - Lead-Based Paint

10.	Section 09954	Painting - Lead-Based Paint

PART 2 - PRODUCTS

2.1	HEPA Filtered Vacuum Cleaners:

A.	Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products which may be incorporated in the Work include, but are not limited to, the following:

2.2	Duct Tape:  Provide 2" (51mm) width tape with an adhesive which is formulated to aggressively stick to sheet polyethylene.

2.3	Wet Detergent Wash:  Provide detergent with a high phosphate content (at least 5%) trisodium phosphate (TSP).  Follow dilution ratio recommended by the manufacturer's instructions.

Wet Detergent Wash:  Provide detergent or cleaning agent formulated to be effective in removing lead dust.  Follow dilution ratio recommended by the manufacturer's instructions.

2.4	Plastic Sheet:  A single polyethylene film in the largest sheet size possible to minimize seams, 4.0 or 6.0 mils thick.

2.5	Barricade Fence:  Lightweight polyethylene or polyethylene - polypropylene blend barricade fence, 4' (1.2m) high in high visibility orange.

PART 3 - EXECUTION

3.1	SECURING WORK AREA:

A.	Secure work area from access by occupants, staff or users of the building.  Accomplish this where possible, by locking doors, windows, or other means of access to the area.

3.2	DEMARCATION OF REGULATED AREA:

A.	Demarcate each exterior Regulated Area with a sheet plastic drop sheet as described below.

B.	Provide barricade fence with support posts 4' (1.22m) o.c.  Provide barrier warning tape at perimeter with the following legend "Caution Lead Hazard  - Do not enter work area unless authorized."  Barricade fence shall be securely fastened and no closer than twelve feet (12') (3.66m) radius from the work.

3.3	EXTERIOR ABATEMENT GENERAL PROCEDURES:

A.	The following precautions and procedures have application to the work of this section.  Workers must exercise caution to avoid the release of lead dust into the air and to contain lead dust and debris on drop sheet.

B.	Before start of work comply with requirement for worker protection in Section 01555 and respiratory protection in Section 01556.

C.	Do not allow eating, drinking, smoking, chewing tobacco or gum, or applying cosmetics in the regulated area.

D.	Provide barricade fencing and signage.  Maintain egress from exits.

E.	Provide one (1) layer of 6 mil polyethylene sheeting as close to foundation as possible.  Extend the sheeting out from the foundation a distance of 3 feet (0.91m) per story being abated or a minimum of 10 feet (3.04m) and a maximum of 24 feet (7.32m).  Weight the sheeting at the foundation and along edges and seams.  Install curbs at perimeter no less than 5" (140mm) high.  Curbs are to minimize the dispersion of lead dust and debris from drop cloth.   Erect vertical shrouds or suspend work if constant wind speed exceeds 15 mph or there is visible movement of debris beyond ground sheeting.

F.	On a daily basis, collect dust and debris by HEPA vacuuming the surface or by wet sweeping.

G.	At the end of each work day, remove polyethylene sheeting  and place in 6 mil disposal bags. Securely store with other waste.

H.	On a daily basis and during final cleanup, visually examine the immediate area to ensure that no lead debris has escaped containment.  Wet sweep or rake up any debris found and place in 6 mil disposal bags.  Securely store with other waste.

I.	Suspend work activities during inclement weather; including but not limited to rain, snow 		ice, and hail.

END OF SECTION - 01505