SECTION 06403 - REPAIR OF LEAD-BASED PAINTED SUBSTRATES

PART 1 - GENERAL

1.1	RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract,

including General and Supplementary Conditions and other Division 1 Specification Sections, apply to this Section.

1.2	SUMMARY OF WORK

A.	This section requires repair of the following lead-

based painted substrates in the areas indicated.

1.	Wood Trim.

2.	Plaster walls and ceilings.

1.3	RELATED WORK SPECIFIED ELSEWHERE

A.	Section 01505 - Exterior Regulated Areas Lead-Based

Paint

B.	Section 01506 - Work Area Containment - Lead-Based

Paint

C.	Section 01555 - Worker Protection - Lead-Based Paint

D.	Section 01556 - Respiratory Protection - Lead-Based

Paint

PART 2 - PRODUCTS

2.1	REPAIR MATERIALS:

A.	Gypsum Wallboard: sized to match existing thickness,

Type: Regular, unless indicated

B.	Joint Compound: Powder or ready-mixed vinyl

C.	Wood Filler:  Epoxy or Latex Based

PART 3 - EXECUTION

3.1	GENERAL

A.	Mist damaged area with wet detergent wash solution.

Using a utility knife, score paint around area to be removed.  Include entire area of patch.

B.	Remove paint from area to be patched with scraper or

utility knife.  Mist area during scraping operations.

C.	Do not allow paint chips, debris or dust to accumulate.

Pick up with wet paper towel and place in disposal bag.

D.	Carefully remove loose paint at edges.  Remove damaged

sections of substrate place in disposal bag.

E.	Prepare area for new patching materials.

1.	Gypsum Board: Cut out area of damaged wall board.

Replace with new section.  Finish edges with joint

compound and dry wall tape.  Do not sand compound, sanding will disturb lead-based paint unless a large area of paint is removed.  Use wet sponge or lightly wet sand joint compound.

2.	Plaster:  Prepare area in accordance with gypsum

board above.  Mix compatible plaster materials and fill area to be patched.  Use wet sponge or lightly wet sand new plaster.

3.	Wood Trim:  Remove damaged area and fill void with

wood filler or epoxy.  finish/lightly wet sand filler:

END OF SECTION  06403