SECTION 09953 - SURFACE PREPARATION-PAINT STABILIZATION-LEAD-BASED

PAINT

PART 1 - GENERAL

1.1	RELATED DOCUMENTS

A.	Drawings and general provisions of the Contract,

including General and Supplementary Conditions and other Division-1 Specification sections, apply to work of this section.

1.2	SUMMARY OF WORK:

A.	Work of this section includes the preparation of surfaces

to be enclosed by the following sections:

1.	06106 -  Wood enclosures - lead-based paint

2.	06107 -  Exterior siding & enclosure - lead-based

paint

3.	09252 -  Gypsum wallboard enclosure - lead-based

paint

B.	Work of this section includes the stabilization of lead-

based painted surfaces to be encapsulated or painted by the following sections:

1.	09253 -  Cementitious Enclosure - Lead-Based Paint

1.	09940 -  Encapsulation - lead-based paint

2.	09954 -  Painting - lead-based paint

PART 2 - PRODUCTS

2.1	Disposal Bags/Plastic Sheeting:  Provide 6 mil polyethylene

disposal bags.

2.2	Wet Detergent Wash:  Provide detergent with a high phosphate

content (at least 5%) trisodium phosphate (TSP).  Follow dilution ratio recommended by the manufacturer's instructions. Wet Detergent Wash:  Provide detergent or cleaning agent formulated to be effective in removing lead dust.  Follow dilution ratio recommended by the manufacturer's instructions.

PART 3 - EXECUTION

3.1	Before starting work of this section, complete the following:

A.	Section 01504 - Temporary Facilities

B.	Section 01505 - Exterior Regulated Areas

C.	Section 01506 - Work Area Containment  (Interior)

D.	Section 01555 - Worker Protection

E.	Section 01556 - Respiratory Protection

3.2	SURFACE PREPARATION

A.	Remove loose, flaking and deteriorated paint by wet

scraping or wet sanding.

B.	Remove loose, unsound or deteriorated substrates.  Place

in 6 mil  polyethylene disposal bag and dispose of in accordance with section 02067 - Disposal of Waste Materials - Lead-Based Paint.

C.	Hepa Vacuum and/or wet wipe to remove all paint chips,

debris and dust generated during the work.  Do not allow dust or debris to accumulate.

D.	Label all areas to be enclosed with permanent marker

every four feet with term "lead paint", in minimum 3" (76 mm) high letters.

3.3	PAINT STABILIZATION

A.	SUBSTRATE REPAIRS

1.	Prior to stabilizing lead-based paint, correct

substrate surface defects.  Remove loose, unsound or deteriorated substrates.  Place in 6 mil polyethylene disposal bag and dispose of in accordance with section 02067 - Disposal of Waste Materials - Lead-Based Paint.

B.	PAINT REMOVAL

1.	Wet Scraping: remove all loose, flaking and

deteriorated paint by wet scraping.  Continually mist surface with water during scraping.

2.	Wet Sanding: prepare finish surface by wet sanding.

Feather edges lightly.  Keep surface wet while sanding.

C.	SURFACE CLEANING

1.	Dust and chips: HEPA vacuum surface after drying.

2.	Chemically treat surface if necessary for good

paint adhesion.  Follow manufacturer's printed instructions for system used.

3.	Test surface for pH.  Place LITMUS paper on wet

surface.  Surface pH should be between 6-8.  Rerinse surfaces that do not meet pH with clear water or weak acid solution.

4.	Remove oils, waxes and mold.  Remove mold with a 1%

to 10% bleach solution.  Provide appropriate eye, skin and respiratory protection during mold decontamination procedures.  Remove waxes with ammonia and water.  Degrease surfaces with suitable cleaner.  Rinse thoroughly following cleaning.

END OF SECTION 09953